Owner:
XXXXXX, XXX



Purchased: 05/30/2004

Vehicle:
Mazda Rx-8 Sport / MT

Color:
Sunlight Silver

VIN:

JM1FE17314010xxxx
General Performance Log

As of 06/14/2004

Gas mileage


Tank #1:
211 miles on 13.1 Gal

[16.1 mpg]   06.11.04


Tank #2:
201 miles on 12.2 Gal

[16.4 mpg]   06.13.04

 DSC
Tank #3:
221 miles on 13.4 Gal

[16.5 mpg]   06.16.04
General Service Requested
As of 06/14/2004

1. ECU code is currently at version “L”. Get “M” version flashed to avoid risk of no-start problem, as stated in TSB #01-001/04.
2. Confirm that vehicle recall service work has been completed.
a. Recall for M/T Dynamic Damper Recall  (NHTSA 04V074000)

b. Recall for Front Passenger side airbag  (NHTSA 04V075000)

Mechanical Log

(Chronological Sort)
Date: June 08, 2004

Mile: 313
Noticed A/C blows warm/cold air intermittently. Confirm VIN range for TSB #07-002/04 and replace A/C Amplifier with part #FEY2-61-540, if applicable.
Date:
June 09, 2004

Mile: 
587

OIL LIGHT came on intermittently while driving at 34-40 MPH in 2nd gear for 3-5 minutes. Request replacement of current oil pan with new part number, per TSB #01-013/04.
Date:
June 10, 2004
Mile:
812

The Navigation system acted erratically during route to new address. It acted as though an input signal was stuck (one of the buttons, or joystick?). It was non-responsive to any input, while it kept beeping, changing modes, recording map points, etc. Condition lasted for 3-5 minutes, until the unit screen went blank and came back with a message “reading data from CD” for about 30 seconds, then resumed normal operation. I saw the “reading data from CD” screen one more time during same trip. Software may need an update.
NAV SYSTEM

Navigation System Versions (normal version page)
DVD 3.18.20.2
Software U9.136 2003.7

ECU Info Page (3 up 3 down 2 up)
Software -> Ver. Info

Main CPU (M) U4.06 2002.9
Main CPU (F) U9.136 2003.7
Sub CPU 82
CD-ROM Drive 2.04
GPS 0.0
Map Database 1.7.27.6
Map Software 1.7.32.3
Date:
June 11, 2004

Mile:
822

Unusually heavy grooves cut into front rotors on both sides of vehicle. Some reports of this from other Rx-8 owners with early VIN.
Date:
June 18, 2004

Mile:
1155

OIL LIGHT on again. 34-40 MPH, 3rd gear traffic.

Recall notices for 2004, Mazda RX-8
Year  :  2004
Vehicle Make  :  MAZDA 
Vehicle Model  :  RX8 

NHTSA Campaign Number  :  04V074000 

Summary  :
AS A RESULT OF AN NCAP TEST, CERTAIN PASSENGER VEHICLES ARE BEING RECALLED FOR IMPROPERLY WIRED FRONT PASSENGER SEAT AIR BAGS. 
Possible Consequences  :
THE AIR BAG WILL NOT PROVIDE ADEQUATE PROTECTION WHEN THE VEHICLE IS INVOLVED IN A FRONTAL CRASH. 
Corrective Action  :
DEALERS WILL INSTALL A NEW WIRING HARNESS IN THE FRONT PASSENGER SEAT AIR BAG SYSTEM. OWNER NOTIFICATION IS EXPECTED TO BEGIN DURING FEBRUARY 2004. OWNERS SHOULD CONTACT MAZDA AT 1-800-222-5500.

NHTSA Campaign Number  :  04V075000 

Summary  :
ON CERTAIN PASSENGER VEHICLES WITH MANUAL TRANSMISSIONS, THE HEAT INSULATOR MAY BE CRACKED CAUSING ABNORMAL NOISE. 
Possible Consequences  :
CONTINUED OPERATION OF THE VEHICLE CAN CAUSE THE INSULATOR TO COME OFF AND COULD POSE A HAZARD TO FOLLOWING VEHICLES. 
Corrective Action  :
DEALERS WILL REPLACE THE HEAT INSULATOR. OWNER NOTIFICATION IS EXPECTED TO BEGIN DURING FEBRUARY 2004. OWNERS SHOULD CONTACT MAZDA AT 1-800-222-5500.
